DOCUMENTATION FOR 24BIT.PRG

Revision 2 / September 7, 1990

The ST uses a 68000 CPU, which has a 24-bit address bus.  That means that the

high-order eight bits of a 32-bit longword used as an address don't matter.

Some programs use those bits to mean something, like the type or size of the

thing a pointer points to.

Those programs are "not 32-bit clean," won't work normally on the TT, because

the 68030 CPU uses all 32 bits of a longword as the address.  If you run

them, you will get bombs or more horrible kinds of system crashes.

Running the program "24BIT.PRG" in your AUTO folder programs the

memory-management unit built into the 68030 so those eight bits are ignored

again.  Some programs which are not 32-bit clean will work after running

24BIT.PRG.

If you run an ST program on your TT and it bombs, you can consider doing

this:

1. Reboot.  It's always a good idea to reboot after a program

bombs, because it might have put things in a funny state and

not cleaned up after itself.

2. Put 24BIT.PRG in your AUTO folder and reboot again so it

executes.

3. Run the program that bombed.  If it works normally, then you

know you have to use 24BIT.PRG before you can run that

program.

Microsoft Write and programs written in compiled GFA BASIC are known to

require the use of 24BIT.PRG.

There are a number of things to keep in mind:

There might be more wrong with a program than not being 32-bit clean.

programs that use the high eight bits of pointers might also play other

tricks which render them incompatible.

24BIT.PRG makes sure the 68030's cache is disabled when it exits, but it is

possible for other programs, like the Desktop or the Control Panel to enable

it again.  You must make sure that the cache is OFF when running programs

which aren't 32-bit clean, because the cache always uses all 32 bits of an

address, even if the memory manager is programmed to ignore them.  Here is an

example of how you can get into trouble:

1. Write $00 to $23XXXXXX.

2. Write $FF to $44XXXXXX (the same address, but with a different

value in the upper eight bits.

3. Read from    $23XXXXXX (the same address as the write in

step 1).  This will read $00, not $FF, because the cache

uses all 32 bits of the address.

You can't access TT RAM using only 24 bits of address, so you can't be using

TT RAM and use this program.  If you have no TT RAM, or if you have some

installed but aren't using it, you can run this program. The program will

tell you if it can't run because there is something in TT RAM.  If you have

TT RAM but aren't using it, the program will run, and arrange it so the

system "forgets" that there is any TT RAM until you reboot.  That is the

price these programs make you pay.

Devices on the VMEbus can not be accessed using only 24 bits of address,

either, so you lose access to them, too, when you run this program. Since

this program can't "allocate" a VMEbus device the way it can TT RAM, there is

no way to protect against this, and the system will probably just crash if

you run a program that access the VMEbus after running this program.

Programs which actually pass 32-bit addresses with nonzero high bytes as

parameters to OS calls can still die, because all 32 bits are written to

(e.g.) the SCSI DMA controller.

24BIT.PRG must run in the AUTO folder because the TT Desktop itself uses TT

RAM if it's available, so 24BIT.PRG must run before the Desktop comes up.  In

addition, 24BIT.PRG must appear in the AUTO folder before anything which

stays resident and has its TT-RAM-allowed bits set; otherwise 24BIT.PRG will

find that thing and refuse to let you into "24-bit mode."

The program 24BIT.PRG must not have its "load in TT RAM" bit set, because

otherwise it will find that TT RAM is not empty and refuse to run.

----------------------------------------------------------------------

Technically speaking, this program rewrites the page tables in memory so bits

31-24 are always mapped to zero.  This way programs which use the high bytes

of pointers can run.

If you have no TT RAM, 24BIT.PRG will program the 68030's memory management

unit and exit without taking up any resources.  It simply re-uses the memory

where the TOS ROM memory-management tables are.

The program will print a message and refuse to run if you aren't on a TT, or

if there is anything in TT RAM.  "Anything in TT RAM" is defined as follows:

the system variables _ramtop and _ramvalid are used to find out if you have

any TT RAM, and if you do, how much.  Mxalloc() is used to allocate all of

it.  If the call fails, it means there's something in there and the program

aborts.

If there's nothing in TT RAM, the program rewrites the page tables and does a

TSR so the TT RAM block it allocated isn't freed.  That way subsequent

programs which have their TT-RAM-enable bits set still won't load there.

This remapping trick will be defeated by programs which somehow install

themselves at the top of TT RAM and bump _ramtop down.  Do not run any of

these if you want to use 24BIT.PRG.  If you write software, don't write

programs that fool with _ramtop, or your users won't be able to use 24BIT.PRG

to run their favorite "unclean" software.