Miki Mouse Monitor (MMM)

Written and performed by:

Don Maple

Postfach 5603

4000 Dusseldorf 1

B. R. Deutschland

Sort of like, well... ahem... documentation really!

1. Once upon a time...

----------------------

Difference between 'backspace' and 'cursor left'. Backspace ($8)

is sent to PIPE (redirected output) while 'cursor left' is just a

screen editing feature.

Similarly 'delete' ($7F) is sent to PIPE while 'insert' is not

because there is no 'insert' ASCII code.

1.1 Screen editing and redirect-active keys

Backspace                - spaces over to the left and sends $8

to redirected output (if present)

Delete                   - deletes character to the left and

drags everything on current line,

when left edge reached it stops, and

sends $7F to redirected output (if

present)

1.2 'Screen only' editing keys

Undo                     - home

Insert                   - inserts a space at current cursor

position and pushes everything on the

line right, when last non-blank

character reaches right edge it stops

ClrHome                  - Clear/Home

(cursor up)              - moves cursor up with wraparound

(cursor down)            - moves cursor down with wraparound

(cursor left)            - moves cursor left with wraparound

(cursor right)           - moves cursor right with wraparound

Tab                      - this key has been reprogrammed to act

as EOL (erase to End Of Line)

1.3 Function keys

Function keys 1-9 can be programmed to act as single stroke  entry

keys.  When a key is pressed the command string associated with it

is displayed and executed.

The command used to set a value to a function key is '!'. Refer to

description of this command in command list for details.

To view the current function key settings press F10.

Example output:

! 1 "M 1000                          "

! 2 "D 2000                          "

...

...

! 9 "X                               "

The  current  settings can be changed by overtyping  and  pressing

[Return]. Maximum string length allowed is 32 characters.

To delete a key either use spaces to blank out the string or enter

a null string ("" - two consecutive double quotes).

One  handy use of the function keys is to assign just  an  address

and then type the command followed by the key.

2. Shake before use...

----------------------

All parameters are in hexadecimal.

Breakpoints are set using TRAP 0. This enables for testing of time

critical routines. However there are two exceptions. One when

multiple itiration breakpoints are specified (B 0 1000 13). This

is accomplished using single step which will corrupt timing. The

other case is when testing routines in ROM. Breakpoints can not be

set on ROM routines. This is not that important because by the

time the program is set to ROMs it has been thoroughly tested. (Right?)

Besides the ROMs can be copied to RAM using 'C' command and  then

tested there.

Upper/lower case is ignored outside single quotes ('). A routine

translates everything on line up until a quote is encountered to

upper case. Only then the command processor starts evaluation.

The  instruction set has been extended to include ILLEGAL  ($4AFC)

instruction as well as LINE-A and LINE-F traps.

[Esc] has been reprogrammed to act as a break key to interrupt

long listings after M, K or D commands. It can also be used to

stop a long H command.

The space bar acts as a 'pause' toggle when using M, K, D, H or $

commands. Pressing the space bar again will 'unfreeze' or continue

the listing.

When redirecting input or output (i and o commands) following  are

the valid device numbers:

0 - centronics

1 - RS-232

2 - Keyboard and screen

3 - MIDI

4 - Keyboard port

Alternatively, a file name can be supplied instead of a device

number to redirect input/output from/to disk.

If output is redirected to a file it will remain active until

any other output redirection occurs when the file will be closed.

If input is redirected from a file it will remain active until the

end of file, when the file will be closed automatically. This file

can  contain commands separated by a CR. The MMM will, however

handle lines ending in CR and LF as well, which is the more common

form used by assorted editors.

3. Yes friends, there is Father Xmas...

---------------------------------------

(Santa Claus for our friends in the colonies...)

In chronological order (i.e. the order of implementation!):

Help                     - Displays list of commands

if pressed after a command it gives

its syntax

x                        - exits back to desktop

m <ffffff> <tttttt>      - displays memory from f to t

m <ffffff>               - displays $100 (256) bytes from f

m                        - displays $100 (256) bytes from current

value of f

[Return]                 - if immediatelly following an 'm'

command same as previous

i <d>                    - redirect input from device d

i filename               - redirect input from file 'filename'

o <d>                    - redirect output to device d

o filename               - redirect output to file 'filename'

f <ffffff> <tttttt> hhhh - fill memory range from f to t with hex

word h

f <ffffff> <tttttt> 'aa  - fill memory range from f to t with 2

ascii bytes a

h <ffffff> <tttttt> hh.. - hunt in memory range f to t for hex

string h, which can be up to 16 bytes

h <ffffff> <tttttt> 'aa.'- hunt in memory range f to t for ascii

string a, which can be up to 32 bytes

c <ffffff> <tttttt> <aaaaaa>

- copy memory from f to t after a

if a < f copy starts from f (up)

if a > f copy starts from t (down)

g <tttttt>               - update registers and then execute

routine from t monitoring for breakpoints

g                        - same as above only execute from the

saved value of PC

[Return]                 - after either of above and after reaching

a breakpoint, continue

j <tttttt>               - JMP to a routine

= $hhhhhhhh              - convert hex to bin and dec

max size 1 longword

= %bbbbbbbbbbbbbbbb...   - convert from bin to hex and dec

max size 32 bits

= ddddd...               - convert from dec to hex and dec

max size 2^31-1

:0012 3456  12 34 56 78..- memory update in hex after 'm',

cursor up and overtype

.00123456 'AaBb~~cc...'  - memory update in ascii after 'k',

cursor up and overtype

Tilda (~) used for unprintables,

and will be ignored as input

k <ffffff> <tttttt>      - displays memory from f to t in ascii

k <ffffff>               - displays $400 (1024) bytes from f

k                        - displays $400 (1024) bytes from current

value of f

[Return]                 - if immediatelly following a 'k'

command same as previous

?                        - displays all registers as follows

0        1        2

,D  00000000 00000000 00000000...      -data registers

,A  00000000 00000000 00000000...      -address registers

,PC 00000000  Status=0300 (T=0 S=0 X=0 N=0 Z=0 V=0 C=0 INT=3) SSP 00...

,d  00000000 00000000... - register update after '?',

cursor up and overtype

d ffffff tttttt          - disassemble from f to t

d ffffff                 - disassemble next 20 instructions

from f

d                        - disassemble next 20 instructions

from current value of f

[Return]                 - if immediatelly following a 'd'

command same as previous

! # "text......"         - assign a string to an  F  key,

where # is key number 1-9,

maximum string length is 32 characters

! # ""                   - delete an F key

l filename               - load file resolving references and

display base page address

l filename ffffff        - load file as is at address f

(for data files, pictures etc)

s filename ffffff tttttt - save file in memory from f to t as is

Subdirectories and drives can be supplied as a part of <filename>.

Ex. l a:\dir1.sub\myfile.prg ...

*                        - display current base page address

(0 means no relocatable files loaded yet)

t ffffff                 - start TRACE (Single step) from f

t                        - TRACE from current PC value (continue)

[Return]                 - after either of the above same as >t<

b <#> bbbbbb             - set specific breakpoint (0-f)

b <#>                    - delete breakpoint

b                        - display all breakpoints

a ffffff                 - start assembly

<                        - save function keys to default file MMM.KEY

< filename               - save function keys to <filename>

>                        - load function keys from default file MMM.KEY

> filename               - load function keys from <filename>

e                        - get (display) current

edit sector buffer address

e ffffff                 - set new edit sector buffer address

r tt ss ii               - read track <t>, sector <s> and side <i>

into edit sector buffer

r                        - read current sector into buffer

w tt ss ii               - write track <t>, sector <s> and side <i>

from edit sector buffer

w                        - write current sector from buffer

$                        - display current path directory

showing the path itself

$ \dir.sub\*.*           - set new path, default drive and display directory

not quite THE END

...to be continued in a galaxy far,far away (yet so close)...

P.S. Oh yeah! This program is (c) Donald P Maple 1986

So no funny business or, as a matter of fact, any business

without first contacting the above named humanoid...